Problem I

Consider sending a large file from a host to another over a TCP connection that has no loss.

- Suppose TCP uses AIMD for its congestion control without slow start. Assuming CongWin increases by 1 MSS every time a batch of ACKs is received and assuming approximately constant round-trip times, how long does it for CongWin to increase from 1 MSS to 6 MSS.

- What is the average throughput (in terms of MSS and RTT) for this connection up through time = 5 RTT.

Problem II

Recall the macroscopic description of TCP throughput. In the period of time from when the connection’s rate varies from W/(2 . RTT) to W/RTT, only one packet is lost (at the very end of the period).

- Show that the loss rate (fraction of packet lost) is equal to:

L=138W2+34W

- Use the result above to show that if a connection has loss rate of L, then its average rate is approximately given by

=1.22⋅MSSRTT⋅L